LEGAL NOTICE NO………………..

THE MINING ACT

(20xx)

1. Citation

2. Interpretation

3. General Administration and Powers

4. Explosive operating plans

5.  General rules for use of explosives

6. Transportation of explosives

7.  Storage of explosives

8. Selling, importing and exporting of explosives

8. Reporting and documentation

9. General safety precautions

10. Training, induction and certificate of competency

11. Inspections

12. Offences and Penalties

IN EXERCISE of the powers conferred by Article 15 of the Mining Act, 20xx, the Minister for Energy and Minerals makes the following Regulations—

MINING (EXPLOSIVES) REGULATONS, 2020

| Citation. | - These Regulations may be cited as the Mining (Explosives) Regulations, 2020. |
| --- | --- |
| Interpretation. | - In these Regulations, unless the context otherwise requires— |
|  | “Act” means the Mining Act, 20xx; “Minister” means the Minister for the time being responsible for matters relating to mining and minerals; “competent person” means a person who in relation to a duty or function has had adequate training and experience that enables that person to perform that duty or function without avoidable danger to that person or other persons; “danger area” means the area inside the fence surrounding an explosives manufacturing plant or explosives magazine, together with related storage area and building, as approved by the Inspector of Mines; “explosive” includes dynamite, black powder, pellet powders, initiating explosives, blasting caps, safety fuse, fuse lighters, fuse igniters, squibs, cordeau detonant fuse, instantaneous fuse, ignite cords, and igniters and any other chemical compound, mixture, or device, the primary or common purpose of which is to function by explosion, which involves substantially instantaneous release of gas and heat, unless that compound, mixture, or device is otherwise specifically classified by the “United Nations Recommendations on Transport of Dangerous Goods, Part 2, Chapter 2.1 “classification of explosives” |
|  | “explosives business” means any commercial activity where explosives and precursor substances for mining or civil works are imported, exported, manufactured, mixed, transported, bought, sold, stored, consumed or destroyed “environmental protection and compliance” means protection and rehabilitation of the environment based on mining and environmental compliance in line with industry best practice and approved by the Minister and/or relevant environmental management authority in Somaliland “mineral right holder” means a holder of a mineral right as provided for in the mining act “Inspector” means Inspector of Explosives or an Inspector of Mines |
| Application and General Administration | - These regulations apply to the importation, export transportation, storage, possession, manufacture and use of explosives for mining, quarrying and civil works as well as substances used for the manufacture of explosives. The general enforcement of compliance with these regulations falls under the Minister as provided for in the Act and in line with best international practices. |
| Explosives operating plans | - (1) A mineral right holder, manager or an authorized service provider shall before conducting any operation under these regulations submit to the Minister an explosives operating plan which shall show in detail the proposed activities to be conducted. - The Minister shall after the approval of the explosives operating plan issue an operating license within thirty days. - A mineral rights holder, manager or service provider shall for the purpose of the renewal of the operating license submit yearly updated explosives operating plans. - A mineral rights holder, manager or service provider shall submit a monthly report to the Minister detailing activities involving importation, exportation, transportation, storage and use of explosives |
| General Rules for use of explosives | - (1) A person shall not handle or use explosives unless that person is competent to do so under these regulations (2) A person who works in a mine shall not take away any explosives from the mine without the written permission of mineral rights holder, manager or service provider. (3) A person shall not smoke any substance or take or send naked light, or machinery likely to generate heat or sparks within the vicinity of a blast site. (4) The minimum safety distance between a blast and a person near the blast site is five hundred meters |
| Transportation of explosives | - (1) A person shall not use a vehicle for transportation of explosives, unless the use of the vehicle for that purpose is authorised by the Inspector. (2) The Inspector shall in the authorisation specify the nature and maximum amounts of explosives to be carried at any time and may impose other conditions that are necessary for the purpose. (3) A person who transports explosives in a motor vehicle shall transport the explosives between 6.00 am and 6 pm (4) A person who transports explosives in a road vehicle shall not drive faster than sixty kilometres per hour. (5) A local authority in consultation with the Minister, may advise on the route to be used by vehicles transporting explosives within its area of jurisdiction. (6) A person driving a vehicle that is transporting explosives shall not stop or park a vehicle in any city, town, village or residential accommodation. (7) A person driving a road vehicle which is transporting explosives shall on approach of a thunderstorm, park the vehicle off the highway at least five hundred meters from an inhabited building |
| Storage of explosives Selling, importation and exporting of explosives General safety procedures Training, induction and certification of competency Inspections | 7. (1) A person in charge of an explosives magazine shall ensure that explosives stored in the magazine are stored in duck boards, runners or shelves in a manner that ensures that the bottom row of cases is not less than one hundred and fifty millimetres from the floor. (2)Explosives shall be Stacked in a magazine to a height of not more than one hundred and eighty centimetres and the height shall be delineated by a red line painted along each of the walls of the storage chambers. (3) The person in charge of an explosives magazine shall give access to the Inspector for inspection of the magazine. (4) A person in charge of a magazine shall assign security guards who are trained/ inducted on basic safety and emergency response to be on duty at the magazine for as long as there are explosives in the magazine. (5) A person in charge of a magazine shall install a proper perimeter wall and proper lighting of the storage area as approved by the Inspector. (6). Explosives shall be stored at designated safe and secure facilities within the nearest security services installastions and shall be released to mineral rights holder, manager or service provider upon written authorization by the minister 8. (1) A person shall not purchase, acquire, or possess explosives without a permit issued under the mining act and these regulations (2) An Inspector shall open and inspect a consignment of explosives imported or exported to determine the quality of the explosives and demonstrate that the product is safe for use. 9. (1) A manager shall prepare general safety rules for activities that relate to explosives and approved by the Minister for which that manager is responsible and copies of these safety rules shall be prominently displayed in a conspicuous place on buildings and installations connected to the explosives (2) The emergency plan is prominently displayed in conspicuous places on buildings and installations connected to the explosives (3) A mineral rights holder, manager or a service provider shall undertake safety, emergency preparedness and response drills at regular intervals in accordance with a plan approved by the minister and in compliance with best practice and set international standards 10. (1) A mineral rights holder, manager or service provider shall utilize training and induction manuals approved by the minister for training and induction of the staff and any visitors to their explosive storage facilities and/or sites where explosives are being used (2) A manager shall ensure that personnel for whom the manager is responsible and who are likely to be involved in an activity relating to explosives receive training in the hazards associated with explosives. (3) Each employee and visitor must receive an induction on the general safety rules. (4) Each employee must receive training relevant to that employee’s job post before that employee assumes job post. (5) All induction and training activities must be documented. 11. The person in charge of an explosives magazine shall give full access to the Inspector or any other person appointed by the Inspector for inspection of the magazine. |
| Offences and penalties. | - In accordance with article 140 of the mining act, any mineral owner and mining company involved in the purchase, transportation, storage, sale and use of explosives who fails to comply with these regulations, commits an offence and is liable on conviction to fine or imprisonment as provided in the Act. |

Dated the………..........…………………………….. 2020

xxxxxxx

Minister, Ministry of Energy and Minerals